| Subnet | Mask | Usable Range | Assigned to | Router Gateway (Last usable) |
| --- | --- | --- | --- | --- |
| 10.29.101.0/26 | 255.255.255.192 | .1 – .62 | R1-ISP LAN | 10.29.101.62 |
| 10.29.101.64/27 | 255.255.255.224 | .65 – .94 | R2-Central Fa0/1 | 10.29.101.94 |
| 10.29.101.96/28 | 255.255.255.240 | .97 – .110 | R2-Central Fa0/0 | 10.29.101.110 |
| 10.29.101.112/28 | 255.255.255.240 | .113 – .126 | R3-ISP LAN) | 10.29.101.126 |
| 10.29.101.128/30 | 255.255.255.252 | .129 – .130 | R1 ↔ R2 WAN | R1: .129 R2: .130 |
| 10.29.101.132/30 | 255.255.255.252 | .133 – .134 | R1 ↔ R3 WAN | R1: .133 R3: .134 |
| 10.29.101.136/30 | 255.255.255.252 | .137 – .138 | R2 ↔ R3 WAN | R2: .137 R3: .138 |

| Subnet Block | CIDR | Subnet Mask | Usable Host Range | Total Hosts | Assigned To | Router Gateway (Last Usable) | Important Devices in this Subnet |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 10.29.101.0 | /26 | 255.255.255.192 | 10.29.101.1 – 10.29.101.62 | 64 | R1-ISP LAN (SW1) | 10.29.101.62 | PC-1A → .1 Home Router (WAN) → .2 |
| 10.29.101.64 | /27 | 255.255.255.224 | 10.29.101.65 – 10.29.101.94 | 32 | R2-Central Fa0/1 (SW2B) | 10.29.101.94 | PSTU_DNS_Server → .93 (second-to-last) |
| 10.29.101.96 | /28 | 255.255.255.240 | 10.29.101.97 – 10.29.101.110 | 16 | R2-Central Fa0/0 (SW2A) | 10.29.101.110 | PC-1B → .97 (first usable) |
| 10.29.101.112 | /28 | 255.255.255.240 | 10.29.101.113 – 10.29.101.126 | 16 | R3-ISP LAN (SW3) | 10.29.101.126 | PSTU_DHCP_Server → .125 PC-1C, Laptop4, Printer → DHCP (.113–.124) |
| 10.29.101.128 | /30 | 255.255.255.252 | 10.29.101.129 – 10.29.101.130 | 4 | WAN: R1-ISP ↔ R2-Central | – | R1-ISP S0/0/0 → .129 R2-Central S0/0/0 → .130 |
| 10.29.101.132 | /30 | 255.255.255.252 | 10.29.101.133 – 10.29.101.134 | 4 | WAN: R1-ISP ↔ R3-ISP | – | R1-ISP S0/0/1 → .133 R3-ISP S0/0/1 → .134 |
| 10.29.101.136 | /30 | 255.255.255.252 | 10.29.101.137 – 10.29.101.138 | 4 | WAN: R2-Central ↔ R3-ISP | – | R2-Central S0/0/1 → .137 R3-ISP S0/0/0 → .138 |

| Device | IP Address | Subnet Mask | Default Gateway | DNS Server |
| --- | --- | --- | --- | --- |
| PC-1A | 10.29.101.1 | 255.255.255.192 | 10.29.101.62 | 10.29.101.93 |
| Home Router (WAN) | 10.29.101.2 | 255.255.255.192 | 10.29.101.62 | 10.29.101.93 |
| PC-1B | 10.29.101.97 | 255.255.255.240 | 10.29.101.110 | 10.29.101.93 |
| PSTU_DNS_Server | 10.29.101.93 | 255.255.255.224 | 10.29.101.94 | – |
| PSTU_DHCP_Server | 10.29.101.125 | 255.255.255.240 | 10.29.101.126 | 10.29.101.93 |
| PC-1C / Laptop4 / Printer | DHCP → 10.29.101.113–.124 | 255.255.255.240 | 10.29.101.126 | 10.29.101.93 |

Step 2 → Task 3: সব রাউটার কনফিগার করো (CLI)

R1-ISP

plaintext

enable

conf t

hostname R1-ISP

enable secret csepstu

line con 0

password csepstu

login

line vty 0 4

password csepstu

login

banner motd #This is lab router R1-ISP. Authorized access only.#

interface FastEthernet0/0

ip address 10.29.101.62 255.255.255.192

no shutdown

interface Serial0/0/0

ip address 10.29.101.129 255.255.255.252

clock rate 56000

no shutdown

interface Serial0/0/1

ip address 10.29.101.133 255.255.255.252

clock rate 56000

no shutdown

ip route 10.29.101.64 255.255.255.128 10.29.101.130

ip route 10.29.101.112 255.255.255.240 10.29.101.134

end

copy running-config startup-config

R2-Central

plaintext

enable

conf t

hostname R2-Central

enable secret csepstu

line con 0

password csepstu

login

line vty 0 4

password csepstu

login

banner motd #This is lab router R2-Central. Authorized access only.#

interface FastEthernet0/0

ip address 10.29.101.110 255.255.255.240

no shutdown

interface FastEthernet0/1

ip address 10.29.101.94 255.255.255.224

no shutdown

interface Serial0/0/0

ip address 10.29.101.130 255.255.255.252

no shutdown

interface Serial0/0/1

ip address 10.29.101.137 255.255.255.252

clock rate 56000

no shutdown

ip route 0.0.0.0 0.0.0.0 10.29.101.129

end

copy run start

R3-ISP

plaintext

enable

conf t

hostname R3-ISP

enable secret csepstu

line con 0

password csepstu

login

line vty 0 4

password csepstu

login

banner motd #This is lab router R3-ISP. Authorized access only.#

interface FastEthernet0/0

ip address 10.29.101.126 255.255.255.240

no shutdown

interface Serial0/0/0

ip address 10.29.101.138 255.255.255.252

no shutdown

interface Serial0/0/1

ip address 10.29.101.134 255.255.255.252

no shutdown

ip route 0.0.0.0 0.0.0.0 10.29.101.133

end

copy run start

SW1

plaintext

conf t

hostname SW1

enable secret csepstu

line con 0

password csepstu

login

line vty 0 4

password csepstu

login

banner motd #This is lab switch SW1. Authorized access only.#

interface vlan 1

ip address 10.29.101.61 255.255.255.192

no shutdown

exit

ip default-gateway 10.29.101.62

end

copy run start

SW2A

enable

conf t

hostname SW2A

enable secret csepstu

line con 0

password csepstu

login

line vty 0 4

password csepstu

login

banner motd #This is lab switch SW2A. Authorized access only.#

interface vlan 1

ip address 10.29.101.98 255.255.255.240

no shutdown

ip default-gateway 10.29.101.110

end

copy running-config startup-config

SW2B

Plaintext

En

conf t

hostname SW2B

SW3

plaintext

En

conf t

hostname SW3